检定/校准证书

Verification / calibration certificate

×××计量测试所

××× Institute of Meterology

科学  公正  准确  高效

本所以法律、法规为准绳，以检定规程、技术标准、技术规范为依据。在检验过程中不受任何干预，维护监测工作的公正性和权威性。

本所从事的检定和检验项目均已取得«计量合格证书»及«社会公用计量标准证书»和«计量认证合格证书»。检定、校准、检验项目的测量均能溯源到国家计量基准，确保量值准确统一。

本所工作人员遵纪守法，廉洁奉公，全心全意为用户服务。欢迎社会各界监督。

Science Justice Accuracy High-efficient

We regard laws and regulations as the criterion,and we take the basis of the examine rules ,technical standard and specifications.In the course of checking,we don’t receive any intervention so that we can ensure the fairness and authoritativeness of checking.

We have already obtained TheCertificate Of Metrology Standard,The M-etrology Standard Certificate Of Public Society and Metrology Authenticate Certificate for all the examination and test project we are engaged on.In order to ensure the  accuracy of the figure ,the project measure of examining,calibrating and inspecting can date from national metrology standard.

We obey the laws,and are honest in performing our official duties ,and werve the people wholeheartedly.Welcome to suoervise us.

×××计量测试所

检定证书

证书编号：            号

送检单位

计量器具名称

型 号 /规 格

出厂编号

制造单位

检定依据

检定结论

批准人

（检定专用章）                核验员

检定员

检定日期             201  年   月   日

有效期至             201  年   月   日

计量检定机构授权证书号：                        号         电    话：

地址：                                                     邮    编：

传真：                                                     电子邮件：

×××计量测试所

证书编号：          号

本单位是依法设置的国家法定计量检定结构，经省质量技术监督局授权。授权证书号为（ ）法计           号。本证书出具的数据通过国家计量检定系统，可溯源至国家计量基准。

检定所使用的计量标准名称：衡器检定装置

测量范围：

不确定度或准确度等级或最大允许误差：M1级

计量标准证书号：

有效期至     201  年6月17日

检定的环境条件：  温度：20℃         湿度：50%RH

检定结果

最大称量Max=15     最小称量Min=100     检定分度值e=d =5

| 检定项目 |  | 法定允差 | 检定结论 |
| --- | --- | --- | --- |
| 外观检查 |  | —— | 符合 |
| 置零准确度 |  | ±0.25e | 合格 |
| 除皮装置准确度 |  | ±0.25e | 合格 |
| 称量测试 | 0e ~500e | 符合 | 合格 |
|  | ﹥500e~2000e | ±1.0e | 合格 |
|  | ﹥2000e~最大称量 | ±1.5e | 合格 |
| 偏载测试 |  | ±1.0e | 符合 |
| 鉴别力测试 |  | 加1.4砝码应有变化 | 有变化 |
| 除皮称量测试 | 0e ~500e | ±0.5e | 合格 |
|  | ﹥500e~2000e | ±1.0e | 合格 |
|  | ﹥2000e~最大称量 | ±1.5e | 合格 |
| 1/2Max重复性 |  | ±1.0e | 合格 |

以下空白

*在填写检定结果时，如需要，可另加附页。

未经本所书面批准，不得部分复印此证书。

×××计量测试所

×××Institute of Meterology

×××计量测试所是国家法定计量检定机构，是鄄城县唯一的计量检定、校准、检验所，建有本县最高社会公用计量标准，具有全县最先进的计量检测设备，拥有高、中级专业技术人员十二人。已开展几何量、力学、温度、电磁学、光学、声学、无线电、时间频率、电离辐射和化学等十大类计量工作。主要承担研究建立社会公用计量标准，进行量值传递，执行国家强制检定、伸裁检定及各种计量器具的检定、校准、商品量和贵金属检验、眼镜质量检验、电能表、水表等四表、出租汽车计价器、医疗设备等多种计量器具的检定、校准和检验工作。承担计量标准考核、计量检定人员技术培训及资格考核、计量技术咨询等任务。本所已通过国家法定计量检定机构考核和计量认证考核，经山东省质量技术监督局授权，出真的证书和报告具有法律效力。

××× Institute of Meterology

××× Institute of measurement and testing is a national legal metrological verification institutions, Juancheng county is the only metrological verification, calibration, inspection, built in the county with the highest public standards of measurement, a measuring equipment of the most advanced county, with high, intermediate professional and technical personnel twenty people. Has carried out geometry, mechanics, temperature, electromagnetism, optics, acoustics, radio frequency, time, ionizing radiation and chemistry and other ten major categories of measurement. Is mainly responsible for the establishment of public standards of measurement, value transfer, the implementation of national compulsory certification, verification and extension cut all measuring instruments verification, calibration, the volume of goods and noble metal inspection, quality test of glasses, electric energy meter, water meter, four meter, taximeter, medical equipment and other measuring instruments verification, calibration and inspection work. Take the examination of measurement standard, the measuring staff technical training and qualification assessment, measurement technical advisory missions. The Institute has passed the national legal metrological verification institutions and the measurement certification examination, authorized by the quality and Technical Supervision Bureau of Shandong Province, the really certificate and report having the force of law.

×××计量测试所

××× Institute of Meterology

地 址：

邮政编码：

查询电话：

传    真：

邮    箱：

Add:

Post Ccode：

Inqulry Tel:

Fax：

E-mall：